SERV. MANUAL FOR   PICO


FC8291/ 02
[image: image1.emf]
PRODUCT INFORMATION  

As usual

TECHNICAL INFORMATION 

Performance
•Input power (IEC): 1400 W
•Suction power (max): 240 W
•Vacuum (max): 20 kPa
•Airflow (max): 26,6 l/s
•Noise level (Lc IEC): 83 dB
Filtration
•Dust capacity: 1.6 l
•Exhaust filter: Micro filter
•Motor filter: Sourcing Pad
Usability
•Action radius: 6 m
•Cord length: 4 m
•Tube type: Plastic 2-P tube
•Handgrip: Ergonomic grip with air slider
•Carrying handle: Top
•Tube coupling: Conical
•Park or storage assist: Vertical and horizontal
•Wheel type: Plastic
•Power control
Nozzles and accessories
•Standard nozzle: Hard floor only nozzle
•Accessories: Crevice tool, Small nozzle
No accessory-storage
Weight and dimensions
•Weight of product: 4 kg
•Dimensions: 337 x 260 x 233 mm
•

Disassembly- and reassembly advise : 
Remove Dustcover by using a screwdriver.
Remove the 2 buttons with a screwdriver.
Remove Tophousing by unscrewing the 3 screws (one is in the parkhook) and unclick the clicknocks
on the the backside.
Unmount the wires from Cordwinder and potmeter.
To repair the PCB, first remove the Potentiometer cover by unscrewing the 3 screws.

[image: image2.jpg]



[image: image3.jpg]


 


Now also Cordwinder and Motor can be taken out and replaced if necessary.

To replace the Dustbag Holder : unclick the knocks and take out.


Partlist :

	1
	996510069772
	DUSTCOVER ASSY

	2
	996510069783
	DBFI ASSY

	3
	996510069773
	DUSTCOVER LOCK

	4
	996510069781
	INLETFILTER FRAME

	5
	432200493971
	MOTOR INLET FILTER PICO

	6
	432200493961
	REUSABLE DUSTBAG (PICO)

	7
	996510069771
	DUSTBAG HOLDER

	8
	996510069776
	ON/OFF BUTTON

	9
	996510069774
	CW BUTTON

	10
	996510069777
	BACKCOVER

	11
	996510069356
	SWITCH

	12
	996510069779
	EXHAUST FILTER FRAME

	13
	432200493681
	EXHAUST MICRO FILTER HARE

	14
	996510069778
	WHEEL ASSY

	15
	432200535401
	CORDWINDER 4M 4.8PIN (PICO)

	16
	432200699601
	KC MOTOR 230V 60HZ MD2 1200W- SERVICE

	17
	996510069782
	BOTTOM HOUSING

	18
	996510069775
	CASTER ASSY

	19
	432200425491
	HOSE ASSY (PICO)

	20
	432200425611
	FIXED 2PCS METAL TUBE

	21
	432200425511
	CREVICE NOZZLE (PICO)

	22
	432200425521
	SMALL NOZZLE (PICO)

	23
	432200425621
	NOZZLE KC92 (32)


